AOS Development Guide

# Contents

1.	GPIO interfaces	1

1.1	Enumerated value of GPIO ports	1

1.2	Service	2

1.3	getSupportedGpioPorts	2

1.4	getGpioValue (String)	3

2.	Wi-Fi interfaces	4

2.1	Permissions	4

2.2	Classes	4

2.2.1	ScanResult	4

2.2.2	WifiConfiguration	5

2.2.3	WifiInfo	5

2.2.4	WifiManager	6

2.3	Wi-Fi hotspot	8

3.	GPS interfaces	11

3.1	GPS positioning	11

4.	RFID interfaces	13

4.1	Programming interface	13

4.2	Parameter interface	14

4.3	Call-back interface	14

4.4	Detail description for interfaces	15

4.4.1	Connect / disconnect RFID	15

4.4.1.1	Connect	15

4.4.1.2	Disconnect	15

4.4.2	18000-6C Commands	16

4.4.2.1	StartRead	16

4.4.2.2	StopRead	16

4.4.2.3	ReadDataByEPC	16

4.4.2.4	ReadDataByTID	17

4.4.2.5	WriteDataByEPC	17

4.4.2.6	WriteDataByTID	18

4.4.2.7	WriteEPCByTID	19

4.4.2.8	Lock	19

4.4.2.9	Kill	20

4.4.3	Reader customized command	21

4.4.3.1	SetCallBack	21

4.4.3.2	SetInventoryPatameter	21

4.4.3.3	GetInventoryPatameter	22

4.4.3.4	GetUHFInformation	22

4.4.3.5	SetRfPower	23

4.4.3.6	SetAntenna	23

4.4.3.7	SetRegion	23

4.4.4	Error code table	24

# 1.	GPIO interfaces

The following permission is required for GPIO operations:

<uses-permission android:name="android.permission.GPIO_PORT"/>

In order to obtain “android.permission.GPIO_PORT”, the provided framework.jar must be imported in Eclipse.

## 1.1	Enumerated value of GPIO ports

(1) GPIO modes: generally, there are 4 working modes for every GPIO port, each mode corresponds to different port function. For general GPIO function, set to mode 0, corresponding to interface set_mode (gpio,mode);

typedef enum {

GPIO_MODE_GPIO  = 0,

GPIO_MODE_00    = 0,

GPIO_MODE_01    = 1,

GPIO_MODE_02    = 2,

GPIO_MODE_03    = 3,

GPIO_MODE_MAX,

GPIO_MODE_DEFAULT = GPIO_MODE_01,

} GPIO_MODE;

(2) GPIO direction: input and output, corresponding to interface set_dir (gpio, dir);

typedef enum {

GPIO_DIR_IN     = 0,

GPIO_DIR_OUT    = 1,

GPIO_DIR_MAX,

GPIO_DIR_DEFAULT = GPIO_DIR_IN,

} GPIO_DIR;

(3) GPIO level: corresponding to interface set_data (gpio, high_or_low);

typedef enum {

GPIO_OUT_ZERO = 0,

GPIO_OUT_ONE  = 1,

GPIO_OUT_MAX,

GPIO_OUT_DEFAULT = GPIO_OUT_ZERO,

GPIO_DATA_OUT_DEFAULT = GPIO_OUT_ZERO,

} GPIO_OUT;

## 1.2	Service

(GpioManager)getSystemService(“gpio”);

## 1.3	getSupportedGpioPorts

This interface is able to obtain which GPIO ports are currently available in the system. Detail configuration is defined in “framework/base/core/res/res/values/config.xml” and it can be modified according to actual application. The purpose for such definition is to avoid misapplication on other system GPIO.

<string-array translatable="false" name="config_gpioPorts">

**************GPIO OUTPUT**************

<item>GPIO19</item>

<item>GPIO82</item>

<item>GPIO66</item>

<item>GPIO83</item>

**************GPIO INPUT**************

<item>GPIO44</item>

<item>GPIO43</item>

<item>GPIO68</item>

<item>GPIO65</item>

</string-array>

## 1.4	getGpioValue (String)

This interface is able to map string and integer.

<integer-array name="config_gpioNums">

<item>19</item>

<item>82</item>

<item>66</item>

<item>83</item>

<item>44</item>

<item>43</item>

<item>68</item>

<item>65</item>

</integer-array>

The following sample code shows how to generate a high level on GPIO19.

@Override

public void onResume() {

super.onResume();

String[] ports = mGpioManager.getSupportedGpioPorts();

if (ports != null && ports.length > 0) {

try {

mGpioPort = mGpioManager.obtainGpioPortControl("GPIO19");

if (mGpioPort != null) {

int gpio_num = mGpioManager.getGpioValue("GPIO19");

mGpioPort.set_mode(gpio_num,0);

mGpioPort.set_dir(gpio_num,1);

mGpioPort.set_data(gpio_num,1);

mLog.setText(mLog.getText() + "set GPIO19 success");

}

else

mLog.setText(mLog.getText() + "set GPIO19 fail");

} catch (IOException e) {

}

}

}

# 2.	Wi-Fi interfaces

## 2.1	Permissions

Wi-Fi associated classes can be found under the Android.net.wifi package. The following permissions are required to use Wi-Fi functions:

- ACCESS_WIFI_STATE

- CHANGE_WIFI_STATE

- CHANGE_WIFI_MULTICAST_STATE

To obtain permissions, the following code will be required to add to the “AndroidManifest.xml” file

<uses-permission android:name="android.permission.ACCESS_WIFI_STATE"></uses-permission>

<uses-permission android:name="android.permission.CHANGE_WIFI_STATE"></uses-permission>

## 2.2	Classes

### 2.2.1	ScanResult

By applying this class, user is able to obtain information of the detected Wi-Fi access point.

An example of output information is shown as below:

The ScanResult contains 5 fields, stated in the following table.

| Return type | Name | Description |
| --- | --- | --- |
| public String | BSSID | The address of the access point. |
| public String | SSID | The network name. |
| public String | capabilities | Describes the authentication, key management, and encryption schemes supported by the access point. |
| public int | frequency | The frequency in MHz of the channel over which the client is communicating with the access point. |
| public int | level | The detected signal level in dBm. |

### 2.2.2	WifiConfiguration

By applying this class, user is able to obtain the network configuration (including security configuration) of a Wi-Fi network. WifiConfiguration contains 6 subclasses, as listed below.

| Subclass | Description |
| --- | --- |
| WifiConfiguration.AuthAlgorthm | Obtain IEEE 802.11 authentication algorithms. |
| WifiConfiguration.GroupCipher | Obtain group ciphers. |
| WifiConfiguration.KeyMgmt | Obtain key management schemes. |
| WifiConfiguration.PairwiseCipher | Obtain pairwise ciphers for WPA. |
| WifiConfiguration.Protocol | Obtain security protocols. |
| wifiConfiguration.Status | Obtain status of a network configuration. |

Refer to Android SDK document for further information about the WifiConfiguration.

### 2.2.3	WifiInfo

By applying this class, user is able to obtain the status information of a currently connected or active Wi-Fi network. Commonly used method is listed in the following table.

### 2.2.4	WifiManager

By applying this class, user is able to mange Wi-Fi connection. Commonly used method is listed in the following table.

The getWifiState() method returns the Wi-Fi enabled state, and the possible returns are listed in the following table.

| Type | Name | Value | Description |
| --- | --- | --- | --- |
| int | WIFI_STATE_DISABLED | 1 | Wi-Fi is disabled. |
| int | WIFI_STATE_DISABLING | 0 | Wi-Fi is currently being disabled. |
| int | WIFI_STATE_ENABLED | 3 | Wi-Fi is enabled. |
| int | WIFI_STATE_ENABLING | 2 | Wi-Fi is currently being enabled. |
| int | WIFI_STATE_UNKNOWN | 4 | Wi-Fi is in an unknown state. |

The following sample code shows how to use Wi-Fi interfaces.

// Open Wi-Fi

public void openWifi(Context context) {

if (!mWifiManager.isWifiEnabled()) {

mWifiManager.setWifiEnabled(true);

}else if (mWifiManager.getWifiState() == 2) {

Toast.makeText(context," Wi-Fi is currently being enabled! ", Toast.LENGTH_SHORT).show();

}else{

Toast.makeText(context,"Wi-Fi is enabled", Toast.LENGTH_SHORT).show();

}

}

// Close Wi-Fi

public void closeWifi(Context context) {

if (mWifiManager.isWifiEnabled()) {

mWifiManager.setWifiEnabled(false);

}else if(mWifiManager.getWifiState() == 1){

Toast.makeText(context," Wi-Fi is currently being disabled! ", Toast.LENGTH_SHORT).show();

}else if (mWifiManager.getWifiState() == 0) {

Toast.makeText(context,"Wi-Fi is disabled", Toast.LENGTH_SHORT).show();

}else{

Toast.makeText(context," close Wi-Fi again",

Toast.LENGTH_SHORT).show();

}

}

## 2.3	Wi-Fi hotspot

Wi-Fi connection and Wi-Fi hotspot can not be enable at the same time. Hence user must switch OFF Wi-Fi connection before using Wi-Fi hotspot.

The following sample code shows how to open and close Wi-Fi hotspot.

private boolean setWifiApEnabled() {

// To open Wi-Fi hotspot, Wi-Fi connection must be switch off

while(mWifiManager.getWifiState() != WifiManager.WIFI_STATE_DISABLED){

mWifiManager.setWifiEnabled(false);

try {

Thread.sleep(200);

} catch (Exception e) {

Log.e(TAG, e.getMessage());

return false;

}

}

// Ensure Wi-Fi hotspot is off

while(getWifiAPState() != WIFI_AP_STATE_DISABLED){

try {

Method method1 = mWifiManager.getClass().getMethod("setWifiApEnabled",

WifiConfiguration.class, boolean.class);

method1.invoke(mWifiManager, null, false);

Thread.sleep(200);

} catch (Exception e) {

Log.e(TAG, e.getMessage());

return false;

}

}

// Open Wi-Fi hotspot

try {

Method method1 = mWifiManager.getClass().getMethod("setWifiApEnabled",

WifiConfiguration.class, boolean.class);

method1.invoke(mWifiManager, null, true);

Thread.sleep(200);

} catch (Exception e) {

Log.e(TAG, e.getMessage());

return false;

}

return true;

}

// Close Wi-Fi hotspot

public void closeWifiAp() {

if (getWifiAPState() != WIFI_AP_STATE_DISABLED) {

try {

Method method = mWifiManager.getClass().getMethod("getWifiApConfiguration");

method.setAccessible(true);

WifiConfiguration config = (WifiConfiguration) method.invoke(mWifiManager);

Method method2 = mWifiManager.getClass().getMethod("setWifiApEnabled", WifiConfiguration.class, boolean.class);

method2.invoke(mWifiManager, config, false);

} catch (NoSuchMethodException e) {

e.printStackTrace();

} catch (IllegalArgumentException e) {

e.printStackTrace();

} catch (IllegalAccessException e) {

e.printStackTrace();

} catch (InvocationTargetException e) {

e.printStackTrace();

}

}

}

# 3.	GPS interfaces

## 3.1	GPS positioning

To obtain permissions, the following code will be required to add to the “AndroidManifest.xml” file

<uses-permission android:name="android.permission.INTERNET"/>

<uses-permission android:name="android.permission.ACCESS_COARSE_LOCATION"/>

<uses-permission android:name="android.permission.ACCESS_FINE_LOCATION"/>

The following sample code shows how to use common GPS interfaces.

// Obtain GPS setting

private void openGPSSettings() {

LocationManager alm = (LocationManager) this

.getSystemService(Context.LOCATION_SERVICE);

if (alm.isProviderEnabled(android.location.LocationManager.GPS_PROVIDER)) {

Toast.makeText(this, "GPS module is working properly",

Toast.LENGTH_SHORT).show();

doWork();

return;

} else {

Toast.makeText(this, "Please Switch On your GPS",

Toast.LENGTH_SHORT).show();

Intent intent = new Intent(Settings.ACTION_SECURITY_SETTINGS);

startActivityForResult(intent, 0); // Return to the GPS inquiring page

}

}

// Obtain longitude and latitude information via GPS

private void doWork() {

String msg = "";

LocationManager locationManager = (LocationManager)

getSystemService(Context.LOCATION_SERVICE);

Criteria criteria = new Criteria();

// To get the best possible positioning information

criteria.setAccuracy(Criteria.ACCURACY_FINE);

criteria.setAltitudeRequired(false);

criteria.setBearingRequired(false);

criteria.setCostAllowed(false);

// Use low power mode

criteria.setPowerRequirement(Criteria.POWER_LOW);

// Obtain the current location provider

String provider = locationManager.getBestProvider(criteria, true);

// Obtain the current location

Location location = locationManager.getLastKnownLocation(provider);

double latitude = location.getLatitude();

double longitude = location.getLongitude();

locationString = "&location=" + latitude + "," + longitude;

keyString = "&key=Yourkey";

questURL = questURL + locationString + keyString;

new ReadJSONFeedTask().execute(questURL);

}

# 4.	RFID interfaces

AOS provides 2 serial COM, the “/dev/ttyMT0” COM is used for RFID, and the “/dev/ttyMT1” is for external communication. In order to perform RFID operation AOS_rfid.jar and serial driver must be imported. Files are located under the “/SDK/libs” folder.

## 4.1	Programming interface

Declaration

import com.rfid.trans.UHFLib;

List of available Interfaces

public int Connect(String ComPort,int BaudRate);

public int DisConnect();

public void SetInventoryPatameter(ReaderParameter param);

public ReaderParameter GetInventoryPatameter();

public int GetUHFInformation(byte Version[],byte Power[],byte band[],byte MaxFre[],byte MinFre[],byte BeepEn[],byte Ant[]);

public int SetRfPower(int Power);

public int SetAntenna(byte AntCfg);

public String ReadDataByEPC(String EPCStr,byte Mem,byte WordPtr,byte Num,byte Password[]);

public String ReadDataByTID(String TIDStr,byte Mem,byte WordPtr,byte Num,byte Password[]);

public int WriteDataByEPC(String EPCStr,byte Mem,byte WordPtr,byte Password[],String wdata);

public int WriteDataByTID(String TIDStr,byte Mem,byte WordPtr,byte Password[],String wdata);

public int WriteEPCByTID(String TIDStr,String EPCStr,byte Password[]);

public int Lock(String EPCStr,byte select,byte setprotect,String PasswordStr);

public int Kill(String EPCStr,String PasswordStr);

public void SetCallBack(TagCallback callback);

public void StartRead();

public void StopRead();

## 4.2	Parameter interface

public class ReaderParameter {

public byte ComAddr; // address of the RFID module.

public int TidPtr; // the entry address of the target TID.

public int TidLen; // the length of the TID, for TidLen = 0 represent EPC reading.

public int Session; // for session is 0 ~ 3 represent S0 ~ S3 respectively. for session = 255 represent auto Session value.

public int QValue; // valid value is 0 ~ 15.

public int ScanTime; // Maximum inventory time

public int Antenna; // Antenna to be used for the inventory, every bit represent 1 antenna

}

## 4.3	Call-back interface

public interface TagCallback {

public void tagCallback(ReadTag tag); // interface for tag call back

public int tagCallbackFailed(int reason); // Error code

}

public class ReadTag {

public String epcId; // tag EPC or TID

public int rssi; // signal strength

public int antId;// involved antenna number, every bit represent 1 antenna. For

instance, 00100000 represent antenna 6.

}

## 4.4	Detail description for interfaces

### 4.4.1	Connect / disconnect RFID

#### 4.4.1.1	Connect

| Definition | int Connect (String serial,int speed); |  |  |
| --- | --- | --- | --- |
| Description | Open the serial COM to connect reader |  |  |
| Parameters | Name | Type | Notes |
|  | serial | string | Name of the serial COM, it will always be “/dev/ttyMT0” for this device |
|  | speed | int | Serial communication baud rate. |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |

#### 4.4.1.2	Disconnect

| Definition | int Disconnect(); |  |  |
| --- | --- | --- | --- |
| Description | Diconnect a specific port and the reader, release the associated resourse |  |  |
| Parameters | Name | Type | Notes |
| Return value (int) | Succeed: 0; Failed: non zero; (for detail error definition, refer to reader error code table). |  |  |

### 4.4.2	18000-6C Commands

#### 4.4.2.1	StartRead

| Definition | Public void StartRead(); |  |  |
| --- | --- | --- | --- |
| Description | Start RFID tags inventory, the obtained tag will be return via the call back interface. |  |  |
| Parameters | Name | Type | Notes |
| Return (void) | N/A |  |  |

#### 4.4.2.2	StopRead

| Definition | Public void StopRead(); |  |  |
| --- | --- | --- | --- |
| Description | Stop the current RFID tags inventory |  |  |
| Parameters | Name | Type | Notes |
| Return (void) | N/A |  |  |

#### 4.4.2.3	ReadDataByEPC

| Definition | public String ReadDataByEPC(String EPCStr,byte Mem,byte WordPtr,byte Num,byte Password[]); |  |  |
| --- | --- | --- | --- |
| Description | Access data banks via EPC mask code |  |  |
| Parameters | Name | Type | Notes |
|  | EPCStr | string | Tag EPC number in hexadecimal |
|  | Mem | byte | Target memory banks. 0 – Password bank: the first 2 bytes are the kill password and the last 2 bytes are the access password. 1 – EPC bank; 2 – TID bank; 3 – User memory bank. |
|  | WordPtr | byte | The entry byte address of the target data |
|  | Num | byte | Length of the data to be read |
|  | Password | byte[] | 4 bytes tag access password |
| Return (String) | If the return string length is 2, it represent error code is returned in this response. Otherwise, tag data is returned |  |  |

#### 4.4.2.4	ReadDataByTID

| Definition | public String ReadDataByTID(String TIDStr,byte Mem,byte WordPtr,byte Num,byte Password[]); |  |  |
| --- | --- | --- | --- |
| Description | Access data banks via TID mask code (the entry address of TID must be 0) |  |  |
| Parameters | Name | Type | Notes |
|  | TIDStr | string | Tag TID number in hexadecimal |
|  | Mem | byte | Target memory banks. 0 – Password bank: the first 2 bytes are the kill password and the last 2 bytes are the access password. 1 – EPC bank; 2 – TID bank; 3 – User memory bank. |
|  | WordPtr | byte | The entry byte address of the target data |
|  | Num | byte | Length of the data to be read |
|  | Password | byte[] | 4 bytes tag access password |
| Return (String) | If the return string length is 2, it represent error code is returned in this response. Otherwise, tag data is returned |  |  |

#### 4.4.2.5	WriteDataByEPC

| Definition | public int WriteDataByEPC(String EPCStr,byte Mem,byte WordPtr,byte Password[],String wdata); |  |  |
| --- | --- | --- | --- |
| Description | Write data to memory banks via EPC mask code |  |  |
| Parameters | Name | Type | Notes |
|  | EPCStr | string | Tag EPC number in hexadecimal |
|  | Mem | byte | Target memory banks. 0 – Password bank: the first 2 bytes are the kill password and the last 2 bytes are the access password. 1 – EPC bank; 2 – TID bank; 3 – User memory bank. |
|  | WordPtr | byte | The entry byte address of the target data location. |
|  | Password | byte[] | 4 bytes tag access password |
|  | wdata | String | Data to be written in hexadecimal, and the length must be a multiple of 4 integer. |
| Return (String) | If the return string length is 2, it represent error code is returned in this response. Otherwise, tag data is returned |  |  |

#### 4.4.2.6	WriteDataByTID

| Definition | public int WriteDataByTID(String TIDStr,byte Mem,byte WordPtr,byte Password[],String wdata); |  |  |
| --- | --- | --- | --- |
| Description | Write data to memory banks via TID mask code (the entry address of TID must be 0) |  |  |
| Parameters | Name | Type | Notes |
|  | TIDStr | string | Tag TID number in hexadecimal |
|  | Mem | byte | Target memory banks. 0 – Password bank: the first 2 bytes are the kill password and the last 2 bytes are the access password. 1 – EPC bank; 2 – TID bank; 3 – User memory bank. |
|  | WordPtr | byte | The entry byte address of the target data location. |
|  | Password | byte[] | 4 bytes tag access password |
|  | wdata | String | Data to be written in hexadecimal, and the length must be a multiple of 4 integer. |
| Return (String) | If the return string length is 2, it represent error code is returned in this response. Otherwise, tag data is returned |  |  |

#### 4.4.2.7	WriteEPCByTID

| Definition | public int WriteEPCByTID(String TIDStr,String EPCStr,byte Password[]); |  |  |
| --- | --- | --- | --- |
| Description | Rewrite EPC number via TID mask code (the entry address of TID must be 0) |  |  |
| Parameters | Name | Type | Notes |
|  | TIDStr | string | Tag TID number in hexadecimal |
|  | EPCStr | string | Tag EPC number to be written in hexadecimal |
|  | Password | byte[] | 4 bytes tag access password |
| Return (String) | If the return string length is 2, it represent error code is returned in this response. Otherwise, tag data is returned |  |  |

#### 4.4.2.8	Lock

| Definition | public int Lock(String EPCStr,byte select,byte setprotect,String PasswordStr); |  |  |
| --- | --- | --- | --- |
| Description | Setup protection status for data banks |  |  |
| Parameters | Name | Type | Notes |
|  | EPCStr | string | Tag EPC number in hexadecimal |
|  | select | byte | 1 byte, target memory bank selection. 0x00 – kill password write protection 0x01 – access password write protection 0x02 – EPC bank write protection 0x03 – TID bank write protection 4 – user bank write protection All other values are reserved. Reader will return a parameter error status in the response frame if other value is delivered in this field. |
|  | setprotect | byte | 1 byte, protection configuration. For Select = 0x00 or 0x01, SetProtect = 0x00 – set to readable/ writable without protection; 0x01 – set to permanently readable/ writable; 0x02 – set to readable/writeable with password protected; 0x03 – set to permanently unreadable/ unwritable. For Select = 0x02, 0x03 or 0x04, SetProtect = 0x00 – set to writable without protection; 0x01 – set to permanently writable; 0x02 – set to writeable with password protected; 0x03 – set to permanently unwritable. |
|  | PasswordStr | string | 4 bytes tag access password |
| Return (String) | If the return string length is 2, it represent error code is returned in this response. Otherwise, tag data is returned |  |  |

#### 4.4.2.9	Kill

| Definition | public int Kill(String EPCStr,String PasswordStr) |  |  |
| --- | --- | --- | --- |
| Description | Tag killing command. After the kill operation, the tags will no longer process any command from reader. |  |  |
| Parameters | Name | Type | Notes |
|  | EPCStr | string | Tag EPC number in hexadecimal |
|  | select | byte | 1 byte, target memory bank selection. 0x00 – kill password write protection 0x01 – access password write protection 0x02 – EPC bank write protection 0x03 – TID bank write protection 4 – user bank write protection All other values are reserved. Reader will return a parameter error status in the response frame if other value is delivered in this field. |
|  | setprotect | byte | 1 byte, protection configuration. For Select = 0x00 or 0x01, SetProtect = 0x00 – set to readable/ writable without protection; 0x01 – set to permanently readable/ writable; 0x02 – set to readable/writeable with password protected; 0x03 – set to permanently unreadable/ unwritable. For Select = 0x02, 0x03 or 0x04, SetProtect = 0x00 – set to writable without protection; 0x01 – set to permanently writable; 0x02 – set to writeable with password protected; 0x03 – set to permanently unwritable. |
|  | PasswordStr | string | 4 bytes tag access password |
| Return (String) | If the return string length is 2, it represent error code is returned in this response. Otherwise, tag data is returned |  |  |

### 4.4.3	Reader customized command

#### 4.4.3.1	SetCallBack

| Definition | public void SetCallBack(TagCallback callback) |  |  |
| --- | --- | --- | --- |
| Description | Setup the call-back interface for an ongoing inventory, tag data will be return via this call-back interface |  |  |
| Parameters | Name | Type | Notes |
|  | callback | TagCallback | Call-back interface for tags |
| Return (void) | N/A |  |  |

#### 4.4.3.2	SetInventoryPatameter

| Definition | public void SetInventoryPatameter(ReaderParameter param) |  |  |
| --- | --- | --- | --- |
| Description | Setup the inventory parameter for an ongoing inventory |  |  |
| Parameters | Name | Type | Notes |
|  | param | ReaderParameter | Inventory parameters |
| Return (void) | N/A |  |  |

#### 4.4.3.3	GetInventoryPatameter

| Definition | public ReaderParameter GetInventoryPatameter() |  |  |
| --- | --- | --- | --- |
| Description | Obtain the inventory parameter for an ongoing inventory |  |  |
| Parameters | Name | Type | Notes |
| Return (ReaderParameter) | ReaderParameter: Inventory parameters |  |  |

#### 4.4.3.4	GetUHFInformation

| Definition | public int GetUHFInformation(byte Version[],byte Power[],byte band[],byte MaxFre[],byte MinFre[],byte BeepEn[],byte Ant[]) |  |  |
| --- | --- | --- | --- |
| Description | Obtain UHF module information. |  |  |
| Parameters | Name | Type | Notes |
|  | Version | byte [] | 2 bytes, represents the firmware version. The 1st byte: the main version number; The 2nd byte: the subversion number. |
|  | Power | byte [] | 1 byte, represents the reader output power. The valid range of Power is 0 ~ 30 and its unit is dBm. |
|  | band | byte[] | 1 byte, represents the frequency band. 1 – "Chinese band2"; 2 – "US band"; 3 – "Korean band"; 4 – "EU band"; 8 – "Chinese band1"; |
|  | MaxFre | byte[] | 1 byte, represent the current maximum working frequency point |
|  | MinFre | byte[] | 1 byte, represent the current minimum working frequency point. |
|  | BeepEn | byte[] | 1 byte, represent the buzzer status (enable/disable). |
|  | Ant | byte[] | 1 byte, the antenna configuration. Every bit represents 1 antenna. |
| Return (String) | If the return string length is 2, it represent error code is returned in this response. Otherwise, tag data is returned |  |  |

#### 4.4.3.5	SetRfPower

| Definition | public int SetRfPower(int Power) |  |  |
| --- | --- | --- | --- |
| Description | Modify the reader RF power |  |  |
| Parameters | Name | Type | Notes |
|  | Power | int | Reader output power. The valid range of Power is 0 ~ 30 and its unit is dBm. |
| Return (String) | If the return string length is 2, it represent error code is returned in this response. Otherwise, tag data is returned |  |  |

#### 4.4.3.6	SetAntenna

| Definition | public int SetAntenna(byte AntCfg) |  |  |
| --- | --- | --- | --- |
| Description | Enable/disable antenna |  |  |
| Parameters | Name | Type | Notes |
|  | AntCfg | byte | Valid antenna number. Bit 0 ~ bit 7 represent antenna 1 to antenna 8 respectively |
| Return (String) | If the return string length is 2, it represent error code is returned in this response. Otherwise, tag data is returned |  |  |

#### 4.4.3.7	SetRegion

| Definition | public int SetRegion(int band,int maxfre,int minfre) |  |  |
| --- | --- | --- | --- |
| Description | Modify antenna working frequency |  |  |
| Parameters | Name | Type | Notes |
|  | band | byte[] | 1 byte, represents the frequency band. 1 – "Chinese band2"; 2 – "US band"; 3 – "Korean band"; 4 – "EU band"; 8 – "Chinese band1"; |
|  | MaxFre | byte[] | 1 byte, represent the maximum working frequency point |
|  | MinFre | byte[] | 1 byte, represent the minimum working frequency point. |

| Return (String) | If the return string length is 2, it represent error code is returned in this response. Otherwise, tag data is returned |
| --- | --- |

Formulae for different frequency bands:

Chinese band2: Fs = 920.125 + N * 0.25 (MHz) where N ∈ [0, 19]. US band: Fs = 902.75 + N * 0.5 (MHz) where N ∈ [0,49]. Korean band: Fs = 917.1 + N * 0.2 (MHz) where N ∈ [0, 31]. EU band: Fs = 865.1 + N*0.2(MHz) where N ∈ [0, 14]. Chinese band1: Fs = 840.125 + N * 0.25 (MHz) where N ∈ [0, 19].

### 4.4.4	Error code table

| Error code | Description |
| --- | --- |
| 0x00 | Operation is successful. |
| 0x01 | Fail to detect tags. |
| 0x05 | Access password error. |
| 0x09 | Kill password error. |
| 0x0A | All-zero tag killing password is invalid. |
| 0x0B | Command is not support by the tag. |
| 0x0C | All-zero tag access password is invalid for such command. |
| 0x0D | Fail to setup read protection for a protection enabled tag. |
| 0x0E | Fail to unlock a protection disabled tag. |
| 0x10 | Some bytes stored in the tag are locked. |
| 0x11 | Lock operation failed. |
| 0x12 | Fail to lock a lock enabled tag. |
| 0x13 | Fail to store the value of some preserved parameters. Configuration will still valid before reader shut down. |
| 0x14 | Modification failed. |
| 0xF8 | Error detected in antenna check. |
| 0xF9 | Operation failed. |
| 0xFA | Tag is detected, but fails to complete operation due to poor communication. |
| 0xFB | No tag is detected. |
| 0xFC | Error code returned from tags. |
| 0xFD | Command length error. |
| 0xFE | Illegal command. |
| 0xFF | Parameter error. |
| 0x30 | Communication error. |